Users can use "USB to TTL232" modify Bluetooth parameters, Use RXD, TXD, GND three signal lines. The baud rate can not be modified, fixed 9600BPS. Parameter support power-down save.

Command Description

3 AT Command

Note：

1 During configuration, 4 Channel Bluetooth relay must be powered

2 Modify the parameters in the Bluetooth unconnected

3 AT Command must be increased Enter(CRLF) key

Command 1: Test Command

| Input Command | Respond | Parameters |
| --- | --- | --- |
| AT | OK | none |

Command 2: Settings- Name

| Input Command | Respond | Parameters |
| --- | --- | --- |
| AT+NAME | +NAME=<Para1> --OK | <Para1>: Bluetooth Name |

For example

Input Command : AT+NAMEeletechsup

Respond :  +NAME=eletechsup

Then the Bluetooth name changed to eletechsup

Command 3: Settings - pairing code

| Input Command | Respond | Parameters |
| --- | --- | --- |
| AT+PIN<Para1> | +PIN=<Para1> --OK | <Para1>: pairing code Default：1234 |

For example

Input Command : AT+PIN0000

Respond :  +PIN=0000

Then the Bluetooth pairing code to 0000，The default pairing code is 1234

NOTE : AT Command must be increased Enter(CRLF) key, Otherwise, without any response