BT25D04 4 Bluetooth relay manual

Product Features:

1 Operating voltage: power supply 1,DC 6.5-25V; power supply 2,DC 5V

2 Operating Current: Standby current (all relays closed) 13MA, 1 relay open 44MA, 2 relays open 76MA, 3 relays open 107MA, 4 relays open 139MA.

3 Bluetooth protocol: Bluetooth Specification V4.0 BLE

4 Based on TI CC2541 chipset, Support android 4.3 or above,Apple 4s or above (IOS8 or above) mobile control

5 2.4GHz-2.48GHz unlicensed ISM band

6 send commands via  Android/Apple Bluetooth APP control relay turned on and off (we provide Bluetooth APP, you can also use your own APP to control relays, sending  commands to control). It can also be triggered by an external pulse signal.

7 "Momentary" "Self-locking" "Interlock" "open" "close" “Delay1(Second)” ,”Delay2(0.1Second) 7 Commands

8 External trigger input, opto-isolated switch, low pulse inputs.

9 size  80*66*19mm

10 Weight : 75g

NOTE:

1 phone must support bluetooth 4.0, such as galaxy S4 (android 4.3),apple 4s(IOS8)

2 Bluetooth 4.0 without matching, so when open bluetooth mobile phone cannot search equipment

3 Without password, open software direct connection

4 Baud rate is 9600BPS, Please do not modify the Bluetooth baud rate, otherwise the relay will not work

Glossary:

NO         : Relay normally open contact

COM       : Relay common contact

NC         : Relay normally closed contact

Momentary  : Mobile terminal input "A0" (B0 / C0 / D0 is the same), the relay is "open",and     after delay of 1 second the relay is "Close".

Self-locking  : Mobile terminal input "A1", the relay is "Open", input "A1", the relay is "Close", and so on

Interlock   : Mobile terminal input "A2", relay 1 is "Open", the other relay is "Close"; input  "B2", relay 2  is "Open", the other relay is "Close", and so on.

Open      : NO connection COM, NC disconnect COM

Close      : NO disconnect COM, NC connection COM

CommandS (Format: character, case-insensitive)

| CommandS | Features | Description |
| --- | --- | --- |
| A0 | CH1 "Momentary" | Mobile terminal input "A0", the relay is "open",and after delay of 1 second the relay is "Close". |
| A1 | CH1 " Self-locking " | Mobile terminal input "A1", the relay is "Open", input "A1", the relay is "Close", and so on |
| A2 | CH1 " Interlock " | Mobile terminal input "A2", relay 1 is "Open", the other relay is "Close"; Input "B2/C2/D2" relay 1 is "Close" |
| A3 | CH1 " Open " | NO1 connection COM1, NC1 disconnect COM1 |
| A4 | CH1 " Close " | NO1 disconnect COM1, NC1 connection COM1 |
| A5****Z | Delay1(Second)”****” Indicates 0001-9999, Z indicates the terminator | e.g.1：Delay 10 seconds, command：A50010Z e.g.2：Delay 999 seconds, command：A50999Z |
| A6****Z | Delay2(0.1Second)”****” Indicates 0001-9999, Z indicates the terminator | e.g.1：Delay 10.5 seconds, command：A60105Z e.g.2：Delay 99.9 seconds, command：A60999Z |
| B0 | CH2 "Momentary" | Mobile terminal input "B0", the relay is "open",and after delay of 1 second the relay is "Close". |
| B1 | CH2 " Self-locking " | Mobile terminal input "B1", the relay is "Open", input "B1", the relay is "Close", and so on |
| B2 | CH2 " Interlock " | Mobile terminal input "B2", relay 2 is "Open", the other relay is "Close"; Input "A2/C2/D2" relay 2 is "Close" |
| B3 | CH2 " Open " | NO2 connection COM2, NC2 disconnect COM2 |
| B4 | CH2 " Close " | NO2 disconnect COM2, NC2 connection COM2 |
| B5****Z | Delay1(Second)”****” Indicates 0001-9999, Z indicates the terminator | e.g.1：Delay 190 seconds, command：B50190Z e.g.2：Delay 9999 seconds, command：B59999Z |
| B6****Z | Delay2(0.1Second)”****” Indicates 0001-9999, Z indicates the terminator | e.g.1：Delay 100.5 seconds, command：B61005Z e.g.2：Delay 999.9 seconds, command：B69999Z |
| C0 | CH3 "Momentary" | Mobile terminal input "C0", the relay is "open",and after delay of 1 second the relay is "Close". |
| C1 | CH3 " Self-locking " | Mobile terminal input "C1", the relay is "Open", input "C1", the relay is "Close", and so on |
| C2 | CH3 " Interlock " | Mobile terminal input "C2", relay 3 is "Open", the other relay is "Close"; Input "A2/B2/D2" relay 3 is "Close" |
| C3 | CH3 " Open " | NO3 connection COM3, NC3 disconnect COM3 |
| C4 | CH3 " Close " | NO3 disconnect COM3, NC3 connection COM3 |
| C5****Z | Delay1(Second)”****” Indicates 0001-9999, Z indicates the terminator | e.g.1：Delay 1 seconds, command：C50001Z e.g.2：Delay 5 seconds, command：C50005Z |
| C6****Z | Delay2(0.1Second)”****” Indicates 0001-9999, Z indicates the terminator | e.g.1：Delay 0.5 seconds, command：C60005Z e.g.2：Delay 199.9 seconds, command：C61999Z |
| D0 | CH4 "Momentary" | Mobile terminal input "D0", the relay is "open",and after delay of 1 second the relay is "Close". |
| D1 | CH4 " Self-locking " | Mobile terminal input "D1", the relay is "Open", input "D1", the relay is "Close", and so on |
| D2 | CH4 " Interlock " | Mobile terminal input "D2", relay 4 is "Open", the other relay is "Close"; Input "A2/B2/C2" relay 4 is "Close" |
| D3 | CH4 " Open " | NO4 connection COM4, NC4 disconnect COM4 |
| D4 | CH4 " Close " | NO4 disconnect COM4, NC4 connection COM4 |
| D5****Z | Delay1(Second)”****” Indicates 0001-9999, Z indicates the terminator | e.g.1：Delay 1111 seconds, command：D51111Z e.g.2：Delay 1015 seconds, command：D51015Z |
| D6****Z | Delay2(0.1Second)”****” Indicates 0001-9999, Z indicates the terminator | e.g.1：Delay 190.5 seconds, command：D61905Z e.g.2：Delay 399.9 seconds, command：D63999Z |

Bluetooth APP operations：

Access Password:1234

Bluetooth Baud rate: 9600BPS

The use of video:

android: http://v.youku.com/v_show/id_XMTM5NTY5MDAyOA==.html

apple : http://v.youku.com/v_show/id_XMTM5NTY4ODI2MA==.html

Typical applications:

1 DC 6-25V control circuit,Wiring diagram below. "LOAD" may be LED lights, fans, motors and

other DC 6-25V equipment

2 DC 5V control circuit,Wiring diagram below. "LOAD" may be LED lights, fans, toy car and

other DC 5V equipment

3 DC 1-100V OR AC 85-265V control circuit,Wiring diagram below. "LOAD" may be LED lights, fans, motors and other DC AC equipment

4 All kinds of motor reversing control: DC 1-100V or AC 85-265V DC/AC Motor