4 Bluetooth relay manual

Product Features:

1: DC 12V power supply (voltage range 9-13V, 5V version can be customized )

2: Standby current (all relays closed) 35MA, 1 relay open 60MA, 2 relays open 91MA, 3 relays open 125MA, 4 relays open 155MA

3: send commands via  Android Bluetooth APP control relay turned on and off (we provide Bluetooth APP, you can also use your own APP to control relays, sending  commands to control)

4 "Momentary" "Self-locking" "Interlock" "open" "close" 5 Commands

Glossary:

NO         : Relay normally open contact

COM       : Relay common contact

NC         : Relay normally closed contact

Momentary  : Mobile terminal input "A0" (B0 / C0 / D0 is the same), the relay is "open",and     after delay of 1 second the relay is "Close".

Self-locking  : Mobile terminal input "A1", the relay is "Open", input "A1", the relay is "Close", and so on

Interlock   : Mobile terminal input "A2", relay 1 is "Open", the other relay is "Close"; input  "B2", relay 2  is "Open", the other relay is "Close", and so on.

Open      : NO connection COM, NC disconnect COM

Close      : NO disconnect COM, NC connection COM

CommandS (Format: character, case-insensitive)

| CommandS | Features | Description |
| --- | --- | --- |
| A0 | CH1 "Momentary" | Mobile terminal input "A0", the relay is "open",and after delay of 1 second the relay is "Close". |
| A1 | CH1 " Self-locking " | Mobile terminal input "A1", the relay is "Open", input "A1", the relay is "Close", and so on |
| A2 | CH1 " Interlock " | Mobile terminal input "A2", relay 1 is "Open", the other relay is "Close"; Input "B2/C2/D2" relay 1 is "Close" |
| A3 | CH1 " Open " | NO1 connection COM1, NC1 disconnect COM1 |
| A4 | CH1 " Close " | NO1 disconnect COM1, NC1 connection COM1 |
| B0 | CH2 "Momentary" | Mobile terminal input "B0", the relay is "open",and after delay of 1 second the relay is "Close". |
| B1 | CH2 " Self-locking " | Mobile terminal input "B1", the relay is "Open", input "B1", the relay is "Close", and so on |
| B2 | CH2 " Interlock " | Mobile terminal input "B2", relay 2 is "Open", the other relay is "Close"; Input "A2/C2/D2" relay 2 is "Close" |
| B3 | CH2 " Open " | NO2 connection COM2, NC2 disconnect COM2 |
| B4 | CH2 " Close " | NO2 disconnect COM2, NC2 connection COM2 |
| C0 | CH3 "Momentary" | Mobile terminal input "C0", the relay is "open",and after delay of 1 second the relay is "Close". |
| C1 | CH3 " Self-locking " | Mobile terminal input "C1", the relay is "Open", input "C1", the relay is "Close", and so on |
| C2 | CH3 " Interlock " | Mobile terminal input "C2", relay 3 is "Open", the other relay is "Close"; Input "A2/B2/D2" relay 3 is "Close" |
| C3 | CH3 " Open " | NO3 connection COM3, NC3 disconnect COM3 |
| C4 | CH3 " Close " | NO3 disconnect COM3, NC3 connection COM3 |
| D0 | CH4 "Momentary" | Mobile terminal input "D0", the relay is "open",and after delay of 1 second the relay is "Close". |
| D1 | CH4 " Self-locking " | Mobile terminal input "D1", the relay is "Open", input "D1", the relay is "Close", and so on |
| D2 | CH4 " Interlock " | Mobile terminal input "D2", relay 4 is "Open", the other relay is "Close"; Input "A2/B2/C2" relay 4 is "Close" |
| D3 | CH4 " Open " | NO4 connection COM4, NC4 disconnect COM4 |
| D4 | CH4 " Close " | NO4 disconnect COM4, NC4 connection COM4 |

Bluetooth APP operations：

Access Password:1234

Bluetooth Baud rate: 9600BPS

Video (shooting bad, just live with) : http://v.youku.com/v_show/id_XMTI5NzczOTY5Ng==.html

Typical applications:

1 DC 12V control circuit,Wiring diagram below. "LOAD" may be LED lights, fans, motors and

other DC 12V equipment

2 DC 12V motor reversing control, Wiring diagram below. "LOAD" can be RS360 and other DC 12V equipment

3 DC 1-48V OR AC 85-265V control circuit,Wiring diagram below(Note:If not DC 12V load, need another DC 12V power supply). "LOAD" may be LED lights, fans, motors and other DC AC equipment

4 DC 1-48V OR AC 85-265V control circuit,Wiring diagram below(Note:If not DC 12V load, need another DC 12V power supply). "LOAD" may be LED lights, fans, motors and other DC AC equipment