Product Name:DC 5-24V 3-5A Flip-Flop Latch DPDT Relay Module Bistable Self-locking Double Switch Board for Arduiuo UNO MEGA AVR LED

Package inlcuded:

1 PCS  3A DC 5V-25V Self-locking (bistable) Flip-Flop Latch DPDT Relay controller

Description:

3A Flip-Flop Latch Self-locking (bistable) DPDT Relay controller

1 Input Voltage: DC 5V(5V version); DC 6-24V(12V version)
2 Module working current:Standby 1-2ma;Relay is on: 60-71MA;
3 Load current: Maximum 5A(Motor <=3A)
4 Power indicator AND Working indicator
5 Control method : low pulse trigger
6 Size: 39.5mm X 33.3mm X 23.3mm
7 Weight: 23 g

This is a bistable (self-locking) relay module,Trigger once, the relay pull (and holds);Trigger again, the relay releases (and holds)


Module interface:
1. V  positive power supply (VCC)
2. G negative power supply (GND)
3. T  : Low pulse trigger(0.2-2 second low pulse trigger)
    
DPDT Relay outputs:
1. NO1: normally open relay interface
2. COM1: Common Interface Relays
3. NC1: normally closed relay interface
4. NO2: normally open relay interface
5. COM2: Common Interface Relays
6. NC2: normally closed relay interface
 


How to use:
 By default, the relay is release, press the trigger button, the relay pull, press again, the relay release, the third press, the relay pull, and so on

Typical applications:
Automotive electronics
Industrial equipment
AC DC Motor
LED lighting
Smart Home / Home Automation
Emergency lighting
MCU AVR UNO MEGE2560 development board
Toy car
PLC

5 wiring methods: 
1 Control two independent loads, the load is activated simultaneously
2 Dual channel simultaneously controls one load (load current multiplication)
3 Control the positive and negative wires (turn off or turn on the positive and negative wires)
4 Polarity switching output (load can be low power motor)
5 Audio 2 L/R to 1 L/R Switcher