Instructions

- Product Features

- DC 12V;

- Relay output isolation;

- Communication interface supports RS232 or TTL serial port;

- Baud rate：9600;

- Communication control: use the provided command to control;

- Relay channel: 8;

- With relay status indicator and power indicator;

- With inching function, the relay can be opened and closed automatically for a period of time by command.

- Product Function

- 8-way relay control;

- RS232 or TTL serial port input control;

- Support the manual control of the upper computer software (the upper computer software is not provided);

- Readable relay on/off status;

- Support self-locking mode (relay state is reversed. If relay no and com are engaged, send this command to disconnect no and com, and vice versa);

- Support inching mode (relay no and com are engaged and disconnected after 200ms);

- Support interlock mode (corresponding relay is closed, other relays are disconnected);

- Support to control single relay on/off operation;

- Support simultaneous control of multi-channel relay on / off operation.

- Parameter

| Relay specification | 10A/250VAC，10A/125AC，10A/30VDC，10A28VDC |
| --- | --- |
| communication interface | RS232，TTL |
| Rated voltage | DC 12V |
| Power indication | 1 way red LED indication |
| Output indication | 8 way red LED indication |
| size | 145*90*40mm |
| weight | With shell 215g; without shell 135g |
| Baud rate | 9600, no parity, 8 data bits, 1 stop bit |

四、Interface Description

- Whole machine interface distribution

- Relay wiring

3、RS232 connection

4、TTL and Power connection

五、Command instructions

Example: Connect to the controller through the USB to RS232 module or USB to TTL module, connect the module to the PC, open the serial debugging assistant, select the corresponding port, select the baud rate 9600, no parity, 8 data bits, 1 stop bit ,as the picture shows.

If you want to open all the relays, send the instruction 55 56 00 00 00 FF 06 AA, 0x5556 is the frame header, 0x000000 is the reserved byte, 0XFF is all relays, 0x06 is the command, which means the multiple relay on/off operation, 0XAA is the end Symbol, the effect of sending instructions is as follows.

The serial port returns 33 3C 00 00 00 FF 00 AA, indicating that all the relays have been turned on. At this time, you can observe that all the relay indicators on the main board are on and the relays are all turned on.

For more instructions, please refer to the " N223B08 8-channel RS232 IO input and output controller command.doc" file.