1 Channel  Multifunction RS22 Relay commamd

This product supports two kinds of instructions, AT command (ASCII code) and 8-byte command (HEX format).

Automatic recognition of two kinds of commamds,no need to switch..

AT command (ASCII code)

Note:AT commands must be uppercase, lowercase invalid.

9600 Band ,8 Data bits,None Parity,1 Stop Bit. This commands has a delay function

Read Status:

Channel 1: AT+R1

Open :

Channel 1 : AT+O1

Close:

Channel 1 : AT+C1

Toggle (Self-locking)

Channel 1: AT+T1

Latch (Inter-locking)

Relay Open : AT+L1

Relay Close : AT+L2

Momentary (Non-locking)

Channel 1: AT+M1

Delay

Channel 1: AT+D1=XXXX

XXXX refers to the 0000 to 9999 figures, Unit is seconds

Return command : OpenX, CloseX (X = 1)

Example 1:

Send command "AT+D1=0010", Channel 1 is "Open", after delay of 10 seconds, channel 1 is "Close"

Send command "AT+D1=0100", Channel 1 is "Open", after delay of 100 seconds, channel 2 is "Close"

Example 2:

Send command "AT+L1", Channel 1 is "Open"

Send command "AT+L2", Channel 1 is "Close"

8-byte command (HEX format)

Note : The data must be in HEX format.

9600 Band ,8 Data bits,None Parity,1 Stop Bit。This commands has no delay function

Control command format:

| Bytes Number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frame head |  | Reserved bytes |  |  | Channel | Command |  | checksum |  |
| Reading status | 0x5556 |  | 0x000000 |  |  | 0x01 Channel 1 | 00 |  | 0xAC |  |
| Relay open | 0x5556 |  | 0x000000 |  |  | 0x01 Channel 1 | 01 |  | 0xAD |  |
| Relay close | 0x5556 |  | 0x000000 |  |  | 0x01 Channel 1 | 02 |  | 0xAE |  |
| Relay toggle | 0x5556 |  | 0x000000 |  |  | 0x01 Channel 1 | 03 |  | 0xAF |  |
| Relay Interlock | 0x5556 |  | 0x000000 |  |  | 0x01 Channel 1 | 04 |  | 0xB0 |  |
| Relay momentary | 0x5556 |  | 0x000000 |  |  | 0x01 Channel 1 | 05 |  | 0xB1 |  |

5 Control commands：

1. Reading status（reading the satus of the relay (on/off)）

Channel 1 ：55 56 00 00 00 01 00 AC

2. Relay open (issue this command ，Relay open , COM connect to NO ）

Channel 1 ：55 56 00 00 00 01 01 AD

3. Relay close (issue this command ，Relay close , COM disconnect NO , and COM connect to NC ）

Channel 1 ：55 56 00 00 00 01 02 AE

4. Relay toggle（Relay status reversal，if COM connect to NO，this commands will Disconnect COM to NO and Reverse COM connect to NC，and vice versa）

Channel 1 ：55 56 00 00 00 01 03 AF

5. Relay Interlock

Channel 1 ：55 56 00 00 00 01 04 B0

6. Relay momentary（Relay COM connect to NO，disconnect after 500MS ）

Channel 1 ：55 56 00 00 00 01 05 B1

Once issue a command, will have a return fame , 7th byte of return fame mean the satus of realy .

Return command format:

| Bytes Number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frame head |  | Reserved bytes |  |  | Channel | Command | checksum |
| Return relay open | 0x333C |  | 0x000000 |  |  | 0x01 Channel 1 | 01 | 0x71 |
| Return all Relay open | 0x333C |  | 0x000000 |  |  | 0X00 All Relay | 01 | 0X70 |

2 return command

1、Return relay open（return this command，mean COM connect to NO ）

Channel 1 ：33 3C 00 00 00 01 01 71

2、Return relay close（return this command，mean COM disconnect NO , and COM connect to NC ）

Channel 1 ：33 3C 00 00 00 01 02 72