1 Channel Multifunction RS232 Relay Manual

Features:

1: Working voltage: 12V

2: (DC12V)Standby current (all relays closed) 16MA, relay open 47MA,

3 Two serial ports: RS232 DB9 interface, TTL 232 interface

4  "Open" "Close" "Momentary" "Self-locking" "Interlock" "Delay"(Only AT command)  6 Commands

5 Two kinds of commands: AT command and 8-byte command, the two commands are automatically recognized, no need to switch.

6: Under the "Delay" command ,the maximum delay is 9999 seconds(Only AT command)

7 Size: 56.5 *31.5 * 17.5mm

8 Weight: 24g

9 Maximum load: 10A / 250VAC, 10A / 125VAC, 10A / 30VDC, 10A / 28VDC, 10A / 12VDC

Glossary:

NO         : Relay normally open contact

COM       : Relay common contact

NC         : Relay normally closed contact

Open       : NO connection COM, NC disconnect COM

Close       : NO disconnect COM, NC connection COM

Momentary : Enter the Momentary command, the Rreceiver Relay is Open, delay of 0.5 seconds after, Relay is Close;

Toggle  : Enter the Toggle command, the Rreceiver Relay is Open, Enter the Toggle command again, Relay is Close;

Latched : Enter the Channel 1 Latched command, the receiver Channel 1 is Open, the Channel 2 is Close.

Enter the Channel 2 Latched command the receiver Channel 2 is Open, the Channel 1 is Close.

Delay : Enter the Delay command, the Rreceiver Relay is Open, delay of 0-9999 seconds(MODBUS command is 0-255 seconds )after, Relay is Close;

During the delay, Eter the Close command, immediately close the relay

For command details, please see 《1 Channel  Multifunction RS232 Relay commamd》

Two serial port wiring methods.  RS232  and TTL 232 wiring

Wiring Diagram:

1 DC 12V control circuit,Wiring diagram below. "LOAD" may be camera,LED lights, fans, motors and other DC 12V equipment

2 DC 1-110VDC 85-265VAC control circuit,Wiring diagram below(Note:If not DC 12V load, need another DC 12V power supply). "LOAD" may be LED lights, fans, motors Lights, fluorescent lights, solar water heaters and other DC AC equipment